# Home Assignment

This assignment is designed to evaluate your ability to analyze real-world data, develop an anomaly detection method, and communicate insights effectively. It simulates a common scenario in industrial condition monitoring where early detection of abnormal behavior is critical.

## Case Description

You are provided with temperature readings from an electric motor that operates continuously (24/7). The motor is equipped with a temperature sensor that records its temperature throughout its operation. Under normal conditions, the motor remains in a healthy state for most of the time. However, any developing issue is expected to manifest as a consistent, gradual increase in temperature.

The dataset contains one year of temperature measurements (from 2024). Your task is to design a method that can detect and alert when a steady upward trend in temperature occurs, indicating a potential problem.

## Expected Output

Your submission should include:

- A Jupyter Notebook containing well-documented code and explanations, prepared for presentation during the interview (approximately 15 minutes).

- Visualizations that support your analysis (Exploratory Data Analysis).

- Relevant statistical analysis of the dataset. Include only those statistics that you believe provide insights for model development or help guide your solution approach.

- A detection method (classifier, algorithm, or model) that assigns each measurement a label:

  - 1 for anomaly (steady increase detected)

  - 0 for normal (standard operation)

- A combined plot showing temperature values over time, with anomalies highlighted as red markers.

- Be prepared to address the following questions during your presentation:

  - What do the plots represent, and how do they support the development of your model?

  - Which statistics did you use, and how do they help in identifying anomalies?

  - What are the strengths and weaknesses of your approach? Specifically, discuss:

    - Detection time versus event start time

    - False positives (incorrect alerts)

    - False negatives (missed anomalies)

## Evaluation Criteria

Your solution will be assessed based on:

- Data Understanding & Preprocessing: How well you explore and prepare the data.

- Model Design & Justification: The logic behind your chosen approach and its suitability for the problem.

- Visualization & Communication: Clarity and relevance of plots and explanations.

- Accuracy & Robustness: How effectively your method detects steady temperature increases while minimizing false alerts.

- Presentation Quality: Ability to explain your reasoning and answer questions clearly.

## Technical Guidelines

- You may use Python and common libraries such as pandas, numpy, matplotlib, scikit-learn, or others as needed.

- Avoid using external APIs or pre-built anomaly detection services.

- Ensure your code is reproducible and runs without errors.

## Submission Instructions

- Submit your Jupyter Notebook and any supporting files as a zipped folder via email before the interview.

- You will be asked to share your screen and present your results during the interview.

- The assignment is expected to take 1–2 days to complete. If you need additional time, feel free to request an extension.